NEUTRON SCATTERING ANALYSIS TOOLKIT nebula3d User & Reference Manual

Three-dimensional reciprocal-space diffuse neutron scattering analysis — powder ring removal, Bragg punch-and-fill, and the 3D-ΔPDF transform, in one reproducible Python pipeline.

Version 0.1.0        •        June 2026

| Language | Python ≥ 3.10 |
| --- | --- |
| License | MIT |
| Core stack | numpy · scipy · h5py · matplotlib |
| Input format | Mantid NeXus (MDHistoWorkspace) |
| Reference data | Mantid NeXus HKL volumes |

Tsung-Han Yang

© 2026 Tsung-Han Yang.  All rights reserved.

# Contents

If the list below is blank after importing into Google Docs, click it and choose “Update table of contents”, or use Insert ▸ Table of contents.

# 1  Introduction

From a raw single-crystal neutron volume to a real-space map of atomic correlations — one structure, four stages, fully reproducible.

nebula3d is a Python 3.10+ toolkit for the complete analysis of three-dimensional (3D) diffuse neutron scattering volumes. It implements a production-ready four-stage pipeline that transforms a Mantid NeXus volume into a real-space 3D difference pair distribution function (3D-ΔPDF).

## 1.1 The Pipeline at a Glance

| 1 | Powder ring removal Subtracts azimuthally smooth powder-ring contamination from the cryostat and sample environment without masking genuine diffuse signal. |
| --- | --- |

| 2 | Bragg punch Masks sharp Bragg and satellite peaks using lattice-aware integer-node detection plus an hkl-agnostic search pass for off-integer satellites. |
| --- | --- |

| 3 | Backfill Replaces masked voxels with physically reasonable estimates from the surrounding radial background or crystal-symmetry equivalents. |
| --- | --- |

| 4 | 3D-ΔPDF transform Converts the cleaned reciprocal-space volume to a real-space pair-correlation map via a correctly centred 3D DFT with apodization and smooth-background subtraction. |
| --- | --- |

KEY IDEA — One structure carries everything Every stage reads and writes a single HKLVolume — the 3D intensity array, per-voxel σ, a validity mask, the HKL axes, and the crystal orientation (UB) matrix travel together from raw input to ΔPDF output.

## 1.2 Scope of This Manual

This manual covers the background physics and mathematics of 3D diffuse scattering analysis, the algorithms and their theoretical basis, a step-by-step workflow, the Python API, every configuration parameter, generic worked examples for single-volume and multi-volume workflows, interactive visualization, known limitations, and a complete reference list.

## 1.3 What nebula3d Does Not Do

The package does not perform raw detector reduction, normalization to an absolute cross-section, or Reverse Monte Carlo (RMC) structural refinement. It assumes the input is a Mantid background-subtracted MDHistoWorkspace volume.

NOTE Input is the *_cc_sub_bkg.nxs file produced by Mantid. For the reduction steps that precede it, use Mantid directly.

# 2  Background Theory

What the measurement contains, and why a Fourier transform of the diffuse part maps disorder into real space.

## 2.1 Neutron Scattering Fundamentals

In a neutron scattering experiment a monochromatic beam with wavevector kᵢ is scattered by the sample. The momentum transfer is:

Q = k_f − k_i ¦Q¦ = 4π · sin θ / λ

For an elastic single-crystal measurement, the scattered intensity I(Q) is recorded over a 3D grid in reciprocal space indexed by fractional Miller indices (h, k, l). The total intensity separates into two parts:

I_total(Q) = I_Bragg(Q) + I_diffuse(Q)

The Bragg contribution is sharp peaks at integer lattice nodes, encoding the average periodic structure. The diffuse contribution is the broad, continuous signal encoding atomic displacements, occupancy disorder, short-range magnetic correlations, and phonon / magnon scattering.

## 2.2 The 3D Difference Pair Distribution Function

The 3D-ΔPDF is the Fourier transform of the diffuse scattering intensity:

Δρ(r) = FT[ I_diffuse(Q) ] = FT[ I_total(Q) − I_Bragg(Q) ]

In real space, Δρ(r) is a 3D map of interatomic-correlation deviations. A positive value at vector r means more interatomic pairs are separated by r than the average structure predicts; a negative value means fewer. The practical relation linking measurement to analysis is:

Δρ(r) ≈ FT[ I_cleaned(Q) ]

where I_cleaned(Q) is the diffuse volume after ring removal, Bragg punching, and backfill.

NOTE — Foundational references Weber & Simonov, Z. Kristallogr. 227, 238–247 (2012); Simonov, Weber & Steurer, J. Appl. Cryst. 47, 2011–2018 (2014). These establish the 3D-ΔPDF formalism and the punch-and-fill data-reduction strategy implemented here.

## 2.3 Coordinate Systems and Conventions

- Fractional HKL.  Reciprocal-lattice coordinates (h, k, l) in units of a*, b*, c*. Integer values are Bragg positions.

- Cartesian Q.  Physical momentum transfer in Å⁻¹, related to HKL by the UB matrix. Magnitude |Q| = 2π/d (physics convention).

- UB matrix.  Combines orientation U and reciprocal metric B. Mantid stores the crystallographic UB (no 2π); nebula3d multiplies by 2π on read.

Q = 2π · UB_cryst · [h, k, l]ᵀ ( Å⁻¹ )

## 2.4 Powder Rings: Physical Origin

Polycrystalline material in the beam path — cryostat, aluminium heat shields, sample holder, capsule walls — produces powder diffraction rings: bands of elevated intensity at fixed |Q|. For aluminium (FCC, a = 4.046 Å), the strongest is Al(111) at |Q| = 2.69 Å⁻¹.

Rings are not isotropic. Detector solid-angle coverage, absorption path length, and normalization artifacts modulate ring intensity with azimuthal direction φ. A useful physical model is:

I_ring(Q, φ) = T(φ) · Σᵢ Aᵢ · G( ¦Q¦ − qᵢ , σᵢ ) I_measured(Q, φ) = I_diffuse(Q) + I_ring(Q, φ)

where G is a Gaussian radial profile for ring i, Aᵢ is the per-ring amplitude, and T(φ) is the shared azimuthal texture.

# 3  Package Architecture

A thin, composable layer over NumPy — one data class, modules per pipeline concern, scripts as entry points.

## 3.1 Module Map

| Module | Responsibility |
| --- | --- |
| nebula3d/core.py | HKLVolume dataclass — the universal data carrier |
| nebula3d/io/ | Mantid NeXus reader, HDF5 load/save, ASCII HKL I/O |
| nebula3d/preprocessing/ | Powder-ring models, radial background, backfill |
| nebula3d/analysis/ | Bragg punch / backfill, 3D-ΔPDF FFT |
| nebula3d/inpainting/ | Symmetry fill, TV (Chambolle–Pock), RBF, biharmonic |
| nebula3d/visualization/ | Slice plots, radial profiles, interactive viewers |
| nebula3d/utils/ | UB matrix, d-spacing, Q↔HKL conversion utilities |
| examples/ | Pipeline scripts and interactive viewer entry points |

## 3.2 The HKLVolume Data Structure

Every stage operates on an HKLVolume dataclass defined in src/nebula3d/core.py:

core.py

@dataclass class HKLVolume: data : NDArray[np.float64] # (nh, nk, nl) intensity sigma : NDArray[np.float64] # (nh, nk, nl) standard deviation mask : NDArray[np.bool_] # True = valid (not masked) h_axis : NDArray # 1D fractional-HKL H coordinates k_axis : NDArray # 1D fractional-HKL K coordinates l_axis : NDArray # 1D fractional-HKL L coordinates ub_matrix : NDArray[np.float64] # (3, 3) physics-convention UB (Å⁻¹) instrument: str # provenance label

| Method | Description |
| --- | --- |
| hkl_grid() | Returns (H, K, L) meshgrids of shape (nh, nk, nl) |
| q_magnitude() | Returns ¦Q¦ in Å⁻¹ for every voxel |
| masked_data() | Returns data with masked voxels set to NaN |
| apply_mask(m) | Updates the validity mask in place |

## 3.3 File Formats

### Input: Mantid NeXus

Raw input is a Mantid MDHistoWorkspace saved as NeXus (*_cc_sub_bkg.nxs). The reader (nebula3d/io/mantid_nxs.py) extracts the signal, variance, mask, bin-edge arrays, and the UB matrix; the crystallographic UB is rescaled by 2π on read.

### Native HDF5

All intermediate and output files are HDF5 (.h5). Load with vol = nebula3d.load(path); save with nebula3d.save(vol, path). The ΔPDF output also stores the transform configuration and direct-lattice constants as HDF5 attributes for the interactive viewers.

# 4  Algorithms

The mathematics and design decisions behind each of the four stages.

## 4.1 Stage 1 — Powder Ring Removal

### 4.1.1  Design Principle

Ring removal is strictly subtractive. The algorithm estimates only the azimuthally smooth ring intensity and subtracts it. It never masks or replaces a voxel merely for showing radial excess — that excess can be genuine anisotropic diffuse scattering.

IMPORTANT — Subtract, never mask Masking radial excess would destroy real diffuse signal. Stage 1 only removes the smooth, φ-modulated ring component; everything anisotropic survives to Stage 2.

### 4.1.2  Non-Parametric Patch Method (production)

The production algorithm (PatchedRadialRingModel) processes one H-slice (0kl plane) at a time, building a non-parametric radial ring model in azimuthal patches:

- Azimuthal patches.  φ = atan2(k_Q, h_Q) ∈ [0, 2π) is divided into N overlapping Hann-weighted patches.

- Robust radial profile.  Within each patch, voxels are binned by |Q| and a trimmed-mean profile rejects Bragg peaks and detector gaps.

- SNIP baseline.  A smooth diffuse baseline is estimated via morphological opening (min then max filters wider than the ring), equivalent to SNIP clipping.

- Ring component.  ring(|Q|) = max(0, profile − baseline); the positive part prevents over-subtraction.

- Subtraction.  Each voxel gets a Hann-weighted blend of ring estimates from the two nearest patches, interpolated to its exact |Q|.

### 4.1.3  Factored Gaussian / SVD Model (legacy)

The older PatchedRingModel fits a rank-1 factored model to the amplitude matrix:

A[i, P] ≈ Aᵢ · T[P]

A rank-1 SVD extracts per-ring amplitudes Aᵢ and patch textures T[P]; a Fourier series gives a smooth periodic T(φ). The rank1_variance diagnostic monitors the approximation — values ≥ 0.90 confirm the shared-texture assumption.

## 4.2 Stage 2 — Bragg Peak Punching

Implemented in nebula3d/analysis/bragg.py by BraggRemover. The recommended production mode is mode="both": a lattice-aware integer-node pass followed by an hkl-agnostic search pass.

### 4.2.1  Integer-Node Path

- Enumerate.  All integer (h, k, l) nodes within the volume are listed.

- Detect.  A local HKL window is inspected; the node is kept only if the local peak exceeds min_intensity and the local median by min_prominence.

- Recentre.  The punch centre moves to the measured local maximum.

- Fit shape (optional).  With integer_optimize_shape, the anisotropic covariance of the peak excess yields per-peak ellipsoid radii, clipped by integer_fit_max_radius_hkl.

- H-slab guard.  integer_h_guard_hkl clips each punch to a slab around the integer-H plane, keeping holes out of diffuse planes at H = ±1/3, ±2/3.

### 4.2.2  Punch Ellipsoid

Each identified peak is punched as an anisotropic ellipsoid in HKL space:

(h−h₀)²/rₕ² + (k−k₀)²/rₖ² + (l−l₀)²/r_l² ≤ 1

where (h₀, k₀, l₀) is the fitted centre and (rₕ, rₖ, r_l) the HKL semi-axes; a guard margin is added to all radii.

### 4.2.3  Search Path (hkl-agnostic)

The search path detects off-integer satellites. At each |Q| shell the background threshold is:

bg_threshold = median(I_shell) + n_mad · MAD(I_shell)

Local maxima above this threshold become punch centres. Structured diffuse planes are protected via search_exclude_h_fractions (periodic, preferred) or search_exclude_h_centers (explicit). For example, fractions [0.3333, 0.6667] protect every H = n ± 1/3 and n ± 2/3 plane across the full H range.

## 4.3 Stage 3 — Bragg-Hole Backfill

### 4.3.1  Q-Shell Fill (recommended)

For ordinary Bragg holes, the robust diffuse level at the same |Q| is the best estimate. For each |Q| bin:

I_fill = median(I_valid) + n_mad · MAD(I_valid)

### 4.3.2  TV Inpainting

For complex cases, Total-Variation inpainting solves:

min_u ½ ‖ W(u − f) ‖² + λ ‖∇u‖₁

where f is the observed data, W the diagonal mask operator, ∇u the 3D forward-difference gradient, and λ the regularisation. The Chambolle–Pock primal-dual algorithm (step sizes τσ = 1/6) preserves piecewise-smooth structures — sharp diffuse sheets and streaks — while suppressing noise.

## 4.4 Stage 4 — 3D-ΔPDF Transform

### 4.4.1  The Correct FFT Recipe

The cleaned volume stores Q = 0 at the array centre (index n//2), but NumPy's fftn expects the origin at [0,0,0]. The correct centred transform of real, centrosymmetric I(Q) is:

Δρ = fftshift( fftn( ifftshift( I_windowed ) ) ).real

Applied step by step:

- Fill masked voxels with 0.

- Optional Q-crop: symmetrically crop to |H| ≤ h_max, |K| ≤ k_max, |L| ≤ l_max.

- Subtract smooth background: I_new = I − GaussianBlur(I, σ) — see 4.4.3.

- Apodize: multiply by a separable Hann / Gaussian window to suppress termination ripples.

- Remove DC: subtract the post-window mean so ΣI = 0 exactly.

- Zero-pad symmetrically to the next power of two, keeping Q = 0 centred.

- Transform: ifftshift → fftn → fftshift.

- Take the real part — valid because I(Q) = I(−Q) for centrosymmetric data.

### 4.4.2  Real-Space Axes

Δh = (h_max − h_min) / (nₕ − 1) freq_h = fftshift( fftfreq(nₕ_padded, d=Δh) ) x_a = freq_h · 2π / ¦UB[:,0]¦ ( Å )

### 4.4.3  Axis-Cross Artifact

A bright cross along the y_K = 0 and z_L = 0 axes can appear in ΔPDF maps. Root cause: a broad separable diffuse envelope whose Fourier transform concentrates energy on the principal axes. Fix: subtract a Gaussian-blurred background before windowing,

I_new = I − GaussianBlur(I, σ ≈ 1.5 r.l.u.)

TIP — Validated default Use SUBTRACT_BG="0,1.5,1.5" — per-axis Gaussian blur with σH = 0 (slice-wise, preserves H-layering) and σK = σL = 1.5 r.l.u. Tune the blur widths for the smooth background in your volume.

### 4.4.4  Centring Bug (fixed 2026-06-05)

Earlier code computed fftshift(fftn(data)) without ifftshift and used one-sided zero-padding. The missing ifftshift introduces a linear phase ramp e^(−iπk) = (−1)ᵏ, flipping the sign of real-space features by pixel parity. Fixed throughout; regression test: test_delta_pdf_centring_positive_peak.

# 5  Installation and Setup

## 5.1 Requirements

| Package | Version | Purpose |
| --- | --- | --- |
| Python | ≥ 3.10 | Core language |
| numpy | ≥ 1.24 | Array operations, FFT |
| scipy | ≥ 1.10 | Interpolation, Gaussian filters |
| h5py | ≥ 3.8 | HDF5 file I/O |
| matplotlib | ≥ 3.7 | Visualization |

## 5.2 Install from Source

bash

git clone https://github.com/user/nebula3d cd nebula3d pip install -e ".[dev]"

## 5.3 Recommended Runtime Environment

bash

export PY=/opt/homebrew/Caskroom/miniforge/base/envs/sci-general/bin/python export PYTHONPATH=src export MPLCONFIGDIR=/private/tmp/nebula3d-mpl

MPLCONFIGDIR keeps the Matplotlib cache outside the repository. If your Python 3.10+ environment already has the dependencies active, replace $PY with python3.

## 5.4 Verifying the Installation

bash

PYTHONPATH=src python -c "import nebula3d; print(nebula3d.__version__)" PYTHONPATH=src python -m pytest -o addopts='' tests/

NOTE The -o addopts='' flag is required because the project runs under conda rather than a virtualenv.

# 6  End-to-End Workflow

One command for the whole pipeline, or run each stage on its own — outputs are cached and skipped when present.

## 6.1 One-Command Pipeline

examples/run_pipeline.py orchestrates all four stages, skipping any whose output already exists for fast incremental reprocessing:

bash

PYTHONPATH=src MPLCONFIGDIR=/tmp/mpl \ python examples/run_pipeline.py

| Environment variable | Effect |
| --- | --- |
| DATA_FILE=/path/to/file.nxs | Override the auto-detected input |
| NO_VIEWER=1 | Skip the GUI; stop after writing _delta_pdf.h5 |
| FORCE=1 | Recompute every stage even if output exists |
| FORCE_FROM=rings¦punch¦backfill¦pdf | Recompute from the named stage onward |
| SLICE_AXIS=H¦K¦L | Direction for ring-removal iteration (default H) |

## 6.2 Stage-by-Stage Commands

Stage 1 · Ring removal

PYTHONPATH=src MPLCONFIGDIR=/tmp/mpl RING_PRESET=cc_on \ python examples/remove_rings_3d.py

Stage 2 · Bragg punch

PYTHONPATH=src MPLCONFIGDIR=/tmp/mpl \ PUNCH_PRESET=cc_on MODE=both MIN_I=0.8 MIN_PROM=0.8 \ INTEGER_FIT_POSITION=1 INTEGER_FIT_SHAPE=1 INTEGER_H_GUARD=0.12 \ SEARCH_EXCLUDE_H_FRACTIONS=0.3333,0.6667 SEARCH_EXCLUDE_H_WIDTH=0.08 \ python examples/punch_bragg_3d.py

Stage 3 · Backfill

PYTHONPATH=src METHOD=q_shell \ python examples/backfill_bragg_3d.py

Stage 4 · 3D-ΔPDF

PYTHONPATH=src MPLCONFIGDIR=/tmp/mpl \ SUBTRACT_BG="0,1.5,1.5" CROP_H=4 CROP_K=8 CROP_L=15 \ APODIZE=gaussian GAUSSIAN_SIGMA=0.4 \ python examples/delta_pdf.py

## 6.3 Multi-Volume Workflow

Run the pipeline once per input volume by pointing DATA_FILE at each raw volume:

bash

# condition A NO_VIEWER=1 DATA_FILE="data/raw/condition_a_cc_sub_bkg.nxs" \ python examples/run_pipeline.py # condition B NO_VIEWER=1 DATA_FILE="data/raw/condition_b_cc_sub_bkg.nxs" \ python examples/run_pipeline.py # condition C NO_VIEWER=1 DATA_FILE="data/raw/condition_c_cc_sub_bkg.nxs" \ python examples/run_pipeline.py

## 6.4 Python API

python

import nebula3d from nebula3d.analysis import BraggRemover, backfill_bragg, compute_delta_pdf vol = nebula3d.load("data/processed/sample_ringremoved.h5") remover = BraggRemover( mode="both", punch_radii=(0.09, 0.12, 0.45), min_intensity=0.8, min_prominence=0.8, integer_optimize_position=True, integer_optimize_shape=True, integer_h_guard_hkl=0.12, integer_local_prominence_n_mad=8.0, search_n_mad=4.0, search_exclude_h_fractions=(1/3, 2/3), search_exclude_h_half_width=0.08, incident_beam_ellipsoid_radii_hkl=(0.15, 0.50, 1.00), ) punched = remover.apply(vol) filled = backfill_bragg(punched, method="q_shell") dpdf = compute_delta_pdf( filled, apodization="gaussian", gaussian_sigma=0.4, crop_hkl=(4, 8, 15), subtract_smooth_bg=(0, 1.5, 1.5), ) nebula3d.save(dpdf, "output_delta_pdf.h5")

# 7  Configuration Reference

Every environment variable, its type, default, and effect — grouped by stage.

## 7.1 Ring Removal

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| RING_PRESET | str | cc_on | Preset: cc_on (conservative) or cc_off (aggressive) |
| Q_STEP | float | 0.02 | Radial bin width for the profile (Å⁻¹) |
| N_FOURIER | int | 8 | Number of Fourier terms for the T(φ) fit |
| PROFILE_METHOD | str | trimmed_mean | Bin statistic: trimmed_mean or median |
| TEXTURE_Q_SMOOTH | float | 0.1 | Smoothing σ for texture along ¦Q¦ (Å⁻¹) |
| SLICE_AXIS | str | H | Axis along which slices iterate: H, K, or L |

## 7.2 Bragg Punch

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| MODE | str | both | Detection: integer, search, or both |
| PUNCH_PRESET | str | cc_on | Preset for radii and thresholds |
| MIN_I | float | 0.8 | Minimum intensity for Bragg detection |
| MIN_PROM | float | 0.8 | Minimum local-median prominence |
| INTEGER_FIT_POSITION | 0/1 | 1 | Optimize punch centre to the measured max |
| INTEGER_FIT_SHAPE | 0/1 | 1 | Fit anisotropic punch radii from data |
| INTEGER_H_GUARD | float | 0.12 | H-slab half-width for integer-node punches |
| SEARCH_EXCLUDE_H_FRACTIONS | str | 0.3333,0.6667 | Periodic H exclusions (fractional parts mod 1) |
| SEARCH_EXCLUDE_H_WIDTH | float | 0.08 | Half-width of search-exclusion slabs |
| INCIDENT_ELLIPSOID_R_HKL | str | 0.15,0.50,1.00 | Direct-beam punch ellipsoid semi-axes (HKL) |

## 7.3 Backfill

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| METHOD | str | q_shell | q_shell, local, tv, symmetry, symmetry+tv |
| Q_SHELL_STEP | float | 0.05 | Bin width for q-shell fill (Å⁻¹) |
| Q_SHELL_MIN_COUNT | int | 10 | Min valid voxels/bin before falling back to local |
| TV_LAM | float | 0.1 | TV regularisation λ (higher = smoother) |
| TV_ITER | int | 300 | Chambolle–Pock iterations |

## 7.4 3D-ΔPDF Transform

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| APODIZE | str | gaussian | Window: hann, gaussian, or none |
| GAUSSIAN_SIGMA | float | 0.4 | Gaussian window width as fraction of Q_max |
| CROP_H/K/L | float | 4/8/15 | Symmetric Q-crop limits in HKL |
| SUBTRACT_BG | str | 0,1.5,1.5 | Per-axis Gaussian-blur σ for background subtraction |
| REAL_SPACE_ANGSTROM | 0/1 | 1 | Real-space axes in Å (1) or HKL units (0) |
| OUT_FILE | str | _delta_pdf.h5 | Output HDF5 path for the ΔPDF volume |

# 8  Visualization & Interactive Exploration

Primitive-first plotting: every function takes an HKLVolume, draws into a Matplotlib Axes, and returns it.

## 8.1 Visualization API

All functions live in nebula3d.visualization and work identically in scripts, IPython, and Jupyter:

python

from nebula3d.visualization import ( extract_slice, plot_slice, plot_radial_profile, plot_azimuthal_map, plot_overview, SliceData, )

## 8.2 plot_slice()

A 2D intensity slice. The plane argument is read as (horizontal, vertical); the remaining axis is cut at value:

| plane | x-axis | y-axis | Fixed axis (cut by value) |
| --- | --- | --- | --- |
| 'kl' / '0kl' | K | L | H |
| 'hl' / 'h0l' | H | L | K |
| 'hk' / 'hk0' | H | K | L |

python

# Log-scale view including the Al(111) ring plot_slice(bkg, "kl", value=0.0, log_scale=True) # Exact fractional plane with manual colour limits plot_slice(data, "hk", value=0.3333, interp=True, vmin=0.0, vmax=0.4)

## 8.3 Radial Profile & Azimuthal Map

python

# Radial profile with an Al(111) ring marker plot_radial_profile(data, mark_q=[2.69]) # Azimuthal texture T(phi) of the Al(111) ring plot_azimuthal_map(data, q_center=2.69)

## 8.4 Interactive Viewers

### Cleanup QA viewer

Four panels — raw → ring-removed → Bragg-punched → backfilled — with an H/K/L plane selector and a cut slider. Use it to confirm integer-H Bragg peaks are removed and fractional-H diffuse planes are preserved.

bash

PYTHONPATH=src MPLCONFIGDIR=/tmp/mpl \ PUNCH_PRESET=cc_on MODE=both MIN_I=0.8 MIN_PROM=0.8 \ INTEGER_FIT_POSITION=1 INTEGER_FIT_SHAPE=1 INTEGER_H_GUARD=0.12 \ SEARCH_EXCLUDE_H_FRACTIONS=0.3333,0.6667 SEARCH_EXCLUDE_H_WIDTH=0.08 \ H_VALUE=0.3333 \ python examples/explore_slice.py

### 3D-ΔPDF orthoslice viewer (recommended)

All three orthogonal real-space planes (x_H–y_K, x_H–z_L, y_K–z_L) with independent cut sliders, a contrast multiplier, and a unit-cell gridline toggle:

bash

PYTHONPATH=src MPLCONFIGDIR=/tmp/mpl \ PDF_FILE=data/processed/condition_a_delta_pdf.h5 RMAX=50 \ python examples/explore_delta_pdf_ortho.py

### Multi-volume comparison grid

A 3×3 grid — rows are three related volumes, columns are the three orthogonal cuts — with shared cut sliders and one global colour scale:

bash

PYTHONPATH=src MPLCONFIGDIR=/tmp/mpl \ PDF_FILES=data/processed/condition_a_delta_pdf.h5,data/processed/condition_b_delta_pdf.h5,data/processed/condition_c_delta_pdf.h5 \ PDF_LABELS="condition A,condition B,condition C" \ python examples/explore_delta_pdf_multi.py

# 9  Worked Examples

## 9.1 Quick Overview of a New Dataset

python

import nebula3d from nebula3d.visualization import plot_overview vol = nebula3d.load("path/to/sample.nxs") fig = plot_overview(vol, log_scale=True) fig.savefig("overview.png", dpi=120)

## 9.2 Locating and Profiling a Powder Ring

python

from nebula3d.visualization import plot_radial_profile, plot_azimuthal_map import matplotlib.pyplot as plt # Identify ring positions fig, ax = plt.subplots() plot_radial_profile(vol, ax=ax, mark_q=[2.69, 4.39, 5.07]) # Check the azimuthal texture T(phi) plot_azimuthal_map(vol, q_center=2.69, q_width=0.05)

## 9.3 Checking Bragg Punch Quality

python

import nebula3d from nebula3d.visualization import plot_slice import matplotlib.pyplot as plt punched = nebula3d.load("data/processed/..._braggpunched.h5") fig, axes = plt.subplots(1, 2, figsize=(12, 5)) # Integer-H: Bragg holes should be punched plot_slice(punched, "kl", value=0.0, ax=axes[0], log_scale=True, title="H=0 (integer)") # Fractional-H: diffuse should be preserved plot_slice(punched, "kl", value=0.3333, interp=True, ax=axes[1], log_scale=True, title="H=1/3 (fractional)") fig.savefig("bragg_check.png", dpi=110)

## 9.4 Computing and Displaying the 3D-ΔPDF

python

import nebula3d from nebula3d.analysis import compute_delta_pdf import matplotlib.pyplot as plt filled = nebula3d.load("data/processed/..._backfilled.h5") dpdf = compute_delta_pdf( filled, apodization="gaussian", gaussian_sigma=0.4, crop_hkl=(4, 8, 15), subtract_smooth_bg=(0, 1.5, 1.5), ) nebula3d.save(dpdf, "output_delta_pdf.h5") hk0 = dpdf.slice_hk0() vmax = abs(hk0.data).max() fig, ax = plt.subplots(figsize=(7, 6)) im = ax.imshow(hk0.data.T, origin="lower", extent=[hk0.x_axis[0], hk0.x_axis[-1], hk0.y_axis[0], hk0.y_axis[-1]], cmap="RdBu_r", vmin=-vmax, vmax=vmax) ax.set_xlabel("x_H (A)"); ax.set_ylabel("y_K (A)") fig.colorbar(im, ax=ax, label="delta-rho (arb.)") fig.savefig("delta_pdf_hk0.png", dpi=150)

# 10  Testing

## 10.1 Running the Suite

bash

# Full suite PYTHONPATH=src python -m pytest -o addopts='' tests/ # Lint python -m ruff check src/ tests/ # Type check python -m mypy src/nebula3d --ignore-missing-imports

## 10.2 Key Regression Tests

| Test | What it verifies |
| --- | --- |
| test_delta_pdf_centring_positive_peak | FFT centring: a positive cosine input gives a positive real-space peak. |
| Ring subtraction (synthetic) | A known Gaussian ring on a flat background is cleanly subtracted. |
| Bragg punch / search modes | Masks match the expected punch radii and guard conditions. |
| Symmetry fill | Inverse-variance-weighted averaging fills masked voxels from Laue equivalents. |
| TV inpainting convergence | The Chambolle–Pock solver converges within the iteration budget. |

# 11  Known Limitations & Troubleshooting

## 11.1 Near-Origin Spike

Symptom.  A very strong feature at r < ~3 Å that dominates the colour scale.

Cause.  Residual high-|Q| Bragg leakage, discontinuities at punch-hole boundaries, and the direct-beam punch.

Fix.  Set colour scales from the p99 of |Δρ| at r > 3 Å — the default in all viewer scripts.

## 11.2 Axis Cross in ΔPDF

Symptom.  A bright cross along the y_K = 0 and z_L = 0 axes in the real-space map.

Cause.  A residual separable diffuse envelope (see §4.4.3).

Fix.  Use SUBTRACT_BG="0,1.5,1.5" (API: subtract_smooth_bg=(0, 1.5, 1.5)).

## 11.3 Residual Ring After Subtraction

Symptom.  A faint ring pattern still visible after ring removal.

Cause.  Gaussian width σᵢ too narrow, texture mismatch (rank1_variance < 0.90), or wrong preset.

Fix.  Widen σᵢ via Q_STEP; try RING_PRESET=cc_off for noisier data; inspect rank1_variance.

## 11.4 Search Mode Punching Diffuse Structure

Symptom.  Structured diffuse on fractional-H planes is partially masked.

Cause.  Search mode does not know the crystal lattice.

Fix.  Add SEARCH_EXCLUDE_H_FRACTIONS=0.3333,0.6667; the periodic form also catches higher-order satellites.

## 11.5 Performance Notes

| Operation | Typical time | Notes |
| --- | --- | --- |
| Ring removal (per-slice) | ~2–5 min | Sequential per H-slice; embarrassingly parallel |
| Bragg punch | ~1–2 min | Scales with the number of peaks |
| Backfill (q-shell) | ~30 sec | Linear in the number of voxels |
| 3D-ΔPDF (FFT) | ~10 sec | Dominated by the FFT of the zero-padded array |

Benchmarks for a 401×401×301 volume (~48 M voxels) on a typical laptop.

# 12  Glossary

| Term | Definition |
| --- | --- |
| Apodization | A window applied to I(Q) before the FFT to suppress termination ripples (Hann, Gaussian). |
| Backfill | Replacement of masked (punched) voxels with estimated background intensities. |
| Bragg peaks | Sharp peaks at integer (and satellite) reciprocal-lattice nodes encoding the average structure. |
| Centrosymmetric | Having inversion symmetry I(Q) = I(−Q); ensures the Fourier transform is real. |
| Diffuse scattering | Broad, continuous intensity encoding disorder, short-range order, and dynamics. |
| Fractional HKL | Reciprocal-lattice coordinates in units of a*, b*, c*; integers are Bragg positions. |
| Hann window | A cosine-squared taper that smoothly suppresses the signal to zero at the Q boundary. |
| HKLVolume | The central data structure: 3D array plus axes, mask, σ, UB matrix, and instrument label. |
| Laue class | Crystal point-group symmetry class (mmm, m3m, 4/mmm…) determining equivalent Q positions. |
| MAD | Median Absolute Deviation; a robust spread measure, median(¦x − median(x)¦). |
| Powder ring | An azimuthally smooth band at fixed ¦Q¦ from polycrystalline material in the beam. |
| Punch | To mask and remove a Bragg peak by setting a region of voxels invalid. |
| Q | Cartesian momentum transfer (Å⁻¹); Q = UB·[h,k,l]ᵀ, ¦Q¦ = 2π/d (physics convention). |
| SNIP | Sensitive Nonlinear Iterative Peak clipping; a morphological 1D baseline method. |
| Total Variation | The regulariser ‖∇u‖₁, promoting piecewise-smooth solutions; preserves sharp sheets. |
| UB matrix | Orientation (U) × metric (B) matrix; Q = UB·[h, k, l]ᵀ. |
| 3D-ΔPDF | Three-dimensional difference pair distribution function; the FT of diffuse scattering. |

# 13  References

| [1] | T. Weber and A. Simonov, "The three-dimensional pair distribution function analysis of disordered single crystals: basic concepts," Z. Kristallogr. 227, 238–247 (2012). DOI: 10.1524/zkri.2012.1504. |
| --- | --- |
| [2] | A. Simonov, T. Weber, and W. Steurer, "Diffuse scattering from the disordered intermetallic compound TbFe₂(Si,Al)₄," J. Appl. Cryst. 47, 2011–2018 (2014). DOI: 10.1107/S1600576714023668. |
| [3] | A. Chambolle and T. Pock, "A First-Order Primal–Dual Algorithm for Convex Problems with Applications to Imaging," J. Math. Imaging Vision 40, 120–145 (2011). DOI: 10.1007/s10851-010-0251-1. |
| [4] | M. Bertalmío, G. Sapiro, V. Caselles, and C. Ballester, "Image inpainting," Proc. ACM SIGGRAPH, 417–424 (2000). DOI: 10.1145/344779.344972. |
| [5] | M. Bertero and P. Boccacci, Introduction to Inverse Problems in Imaging, IOP Publishing, Bristol (1998). ISBN: 978-0750304351. |
| [6] | Mantid Project, "Mantid — Manipulation and Analysis Toolkit for Instrument Data," (2013–present). www.mantidproject.org. |
| [7] | C. G. Ryan et al., "SNIP, a statistics-sensitive background treatment for the quantitative analysis of PIXE spectra," Nucl. Instrum. Methods B 34, 396–402 (1988). |